LAB: Analog Input using ADC Part 2

- Line Tracking with IR Reflective Sensors-

I. Overview

In this lab, we will learn how to configure and use ‘Analog mode’ of GPIO input of the MCU. Then, we will learn how to triggering ADC by using a timer, to control the sampling period of ADC measurements. From acquiring measurements from IR sensors, you are required to create a simple logic to follow a white line on the black track to control your mobile platform.

The objectives of this lab are learning how to

- Read and configure registers of ADC

- Create your own functions for configuring and reading ADC

- Use external timers to trigger ADC

Preparation:

- TCRT 5000 IR Reflective sensors. Read more about the sensor especially input and output voltage.

- You need to study the following registers: ADC register in ‘STM Reference Manual pg. 212 - 239’’

Figure 1. IR Reflective Sensor

II. Pre-Lab

A. ADC Register

- List of ADC registers for this and previous LAB (ADC Part I and ADC Part II)

| Type | Register Name | Description |
| --- | --- | --- |
| RCC | RCC_AHB1ENR | Enable the clock of GPIO Port |
|  | RCC_APB2ENR | Enable the clock of ADC |
| GPIO | GPIOx_MODER | Analog |
|  | GPIOx_PUPDR | No Pull-Up, No Pull-Down: |
| ADCx | ADCx_ CCR | Select the prescaler of ADC clock frequency |
|  | ADCx_CRx | Start one ADC conversion with resolution setting |
|  |  | Choose the data alignment (right/left). |
|  | ADCx_SQRx | Configure the sequence of conversation |
|  | ADCx_SMPRx | Decide the sampling time of conversation |
|  | ADCx_SR | Read the EOC (end of conversion) bit |
|  | ADCx_DR | Read the converted data from ADC pin |

B ADC Register for External Trigger

- Registers for ADC Trigger Output(TRGO)

| Type | Register Name | Description |
| --- | --- | --- |
| TIMx | TIMx_CR2->MMS[2:0] | Selects Triger Output in timer |
| ADCx | ADC_CR2->EXTSEL[3:0] | select which out of 16 possible events can trigger conversion for the regular and injected groups |
|  | ADC_CR2->JEXTSEL[3:0] |  |
|  | ADC_CR2->EXTEN [1:0] | Trigger detection disable Detection on rising/falling/rising and falling edge |
|  | ADC_CR2->JEXTEN [1:0] |  |

- External Trigger

  - ADC conversion can be triggered by (1) Software (2) External

  - Software Trigger:  ADC_CR->ADSTART

  - Hardware signals such as TIMx can be used to trigger ADC conversions

  - By choosing a timer trigger, we can configure the sampling time of ADC conversion

  - Trigger Source: EXTSEL is used to select which trigger event to use

  - Trigger Polarity: EXTEN is used to select the edge of trigger

    - If EXTEN[1:0]=00,  select software trigger. Otherwise choose hardware trigger

- ADC block diagram with External Trigger

- Process of configuring External Trigger (e.g. TIM4)

GPIO Pin setting ADC setting // same as previous lab - Enable ADC peripheral clock (RCC_APB2ENR) - Configure clock prescaler. (ADC_ CCR) - Configure ADC data resolution and alignment. (ADC1_CR1, CR2) - Enable continuous conversion mode (ADC1_CR2) - Configure the number of conversions. (ADC1_SQR1) - Configure the channel and sequence of conversion. (ADC1_SQRn) - Configure the sampling time with choosing the number of cycles. (ADC1_SMPRn) // External Trigger - Select Trigger Source (ADC1_CR->EXTSEL) - Select Trigger Polarity (ADC1_CR->EXTEN) - Enable ADC and start conversion. (ADC1_CR2) TIMx setting - Enable Timer Clock - Counting Direction Setting - Master Mode Selection Bits MMS[2:0]: choose Trigger output (TRGO) - Output Compare Mode - Set PSC, ARR - Enable Counter

B. Register Setting

1. GPIO Pin Initialization

- ADC pin: Port A pin 0 / Analog / No pull-up & No pull-down.

- Read appendix for ADC pin map

2. ADC Initialization for External Trigger

Common Setting

- ADC Intialization: Prescaler /2, 12-bit resolution, right alignment, continuous conversation mode, one channel scan in regular group, channel sample time selection 84-cycles

- Use your own functions created in previous lab for GPIO configuration. See (LAB: ADC Part I)

External Trigger Source Selection

- ADC1_CR2: External event by Timer3 TRGO event

Register map goes here

| Register | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mask |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Value |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- ADC1_CR2: Trigger Detection on Rising Edge

| Register | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |  |  |  |  | 11 | 10 | 9 | 8 |  |  |  |  |  |  | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mask |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Value |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

3. TIMx Initialization for External Trigger for ADC

Common Setting

- TIM3 Intialization: Output compare mode, up-counter, PWM mode_1,  1 KHz,

- Use your own functions created in previous lab for Timer configuration. See (LAB: Timer/PWM)

Timer Trigger Setting (TIMx_CR2)

- Master Mode Selection ( MMS[1:0] ):  Configure master mode of timer for synchronization (TRGO)

- For selecting Compare(OC1REF to OC4REF) for ADC trigger,  choose MMS[2:0]=100~111

- TIMx_CR2: Timer master mode- Compare OC1REF for TRGO

| Register | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 |  |  |  |  | 11 | 10 | 9 | 8 |  |  |  |  |  |  | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mask |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Value |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

IV. Exercise

- Modify your own functions of ADC and Timer initialization for ADC External trigger mode

- Modify TIM3 initialization to select TRGO as OC1REF. Set it as 1KHz by default, or you can give the timer period.

- Modify ADC initialization to be triggered by Timer 3. Change the sampling time of ADC by changing the Timer 3 period.

- You have used polling method to read ADC in the previous lab. Modify the program to read the ADC data by using interrupt.

- Make sure that any interrupt handler should be as concise as possible.

- Example code:

- Create a simple program to control RC Car to trace a line using the IR sensors

- You can choose any ADC pins and choose an appropriate ADC clock frequency

- Set the ADC sampling rate to be below 1KHz, to decrease burden to your CPU.

- Use the analog pin from the IR sensor modules and determine threshold value to differentiate dark and white surface of target

- You can use either regular or injected groups

- Create a logic to trace a white line on dark background surface for your RC car. You will use it in your final project.

Appendix

- Pin Configuration of NUCLE-F411RE

- ADC GPIO pinout for STM32f411