LAB: GPIO Digital I/O

- LED Toggle with Push-Button -

I. Overview

In this lab, we will learn how to control digital I/O of GPIOs of the MCU board to turn on/off an LED with a push-button input. The LED should be turned on when the button is pressed, and vice versa.

The objectives of this lab are learning how to

- Read and configure registers of digital GPIO of MCU

- Program firmware to control digital input/output pins

- Create your own functions for GPIOs

Preparation:

- You need to read about the following registers: GPIO, ‘STM Reference Manual pg. 145-163’

- Install Keil uVision

II. Pre-Lab

A. Bit Operation

- Fill in the blanks. You should write the answer in both hexa-decimal and binary number.

|  | Bit operation | Description | Answer |
| --- | --- | --- | --- |
|  | 0xA & 0x2 | AND |  |
|  | 0x3 ¦ 0xC | OR |  |
|  | ~0x7 | NOT |  |
|  | 0xF ^ 0x5 | XOR(Toggle) |  |
|  | 0x1 << 3 | Shift left |  |
|  | 0xC >> 2 | Shift right |  |
|  | 0x11 ¦= 1<<3 | Set bit |  |
|  | 0xFF &= ~1<<4 | Clear bit |  |
|  | val=0x0F &1<<3 | Read bit |  |

B. GPIO Register

- List GPIO registers for this LAB

| Type | Register Name | Description |
| --- | --- | --- |
| GPIO | GPIOx_MODER | Mode: Output/Input/Analog |
|  | GPIOx_OTYPER | Output Type: Opendrain/Push-Pull |
|  | GPIOx_OSPEEDR | Output Speed: |
|  | GPIOx_PUPDR | Pull-Up Pull-Down: |
|  | GPIOx_IDR | Input Data Register |
|  | GPIOx_ODR | Output Data Register |

- Schematic

- Process of GPIO register initiation

0. Enable Peripheral Clock (AHB1ENR) 1. Configure as Digital Output or Input (GPIOx_MODER) 2. Configure pull-up/down resistors (PUPDR) 3. For Output: Configure Output Type (OTYPE) 4. For Output: Configure Output Speed (OSPEEDR) 5. Read Data or Output Data (ORD / IDR)

C. Register Setting

1. Pin Initialization & Set LED: Port A Pin 5 / Output / Push-Pull / No Pull-Up& No Pull-Down

- GPIOx_MODER: Output

| Register | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mask |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 | 1 |  |  |  |  |  |  |  |  |  |  |
| Value |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 | 1 |  |  |  |  |  |  |  |  |  |  |

GPIOA_MODER |= 3<<(5*2)

- GPIOx_OTYPER: Push-Pull

| Register | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mask |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |
| Value |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |  |  |  |  |  |

GPIOA_OTYPER &= ~(1<<5)

- GPIOx_PUPDR: no pull-up no pull-down

Register map goes here

| Register | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mask |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Value |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

GPIOA_PUPDR  =

- GPIOx_ODR: Set LED

Register map goes here

| Register | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mask |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Value |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Code goes here

2. Pin Initialization & Read PushButton: Port C Pin 13 / Input // Pull-Up

- GPIOx_MODER: Input

| Register | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mask |  |  |  |  | 1 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Value |  |  |  |  | 1 | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

GPIOC_MODER |= 3<<(13*2)

- GPIOx_PUPDR: pull-up

Register map goes here

| Register | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mask |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Value |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

GPIOC_PUPDR  =

- GPIOx_IDR: Read Push-Button Value

Register map goes here

| Register | 31 | 30 | 29 | 28 | 27 | 26 | 25 | 24 | 23 | 22 | 21 | 20 | 19 | 18 | 17 | 16 | 15 | 14 | 13 | 12 | 11 | 10 | 9 | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Mask |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Value |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Code goes here

III. Tutorial

A. Mbed online compiler

- Open https://www.mbed.com/en/

- Click on Compiler

- Create new program. If it asks for ‘add new platform’

then  ‘Add Platform’  search for ‘NUCLEO-F411RE’ board   Click ‘Open Mbed Complier’

- Then, create new program as ‘ Tutorial1_LED’

- Open ‘main.cpp’ and delete the example codes.

- Write the following code on ‘main.cpp’.

- To compile the program, click on ‘Compile’ button

- Then, the binary file “Tutorial1_LED.NUCLEO_F411RE.bin” of the project will be created and downloaded on your computer.

- Connect the MCU board to your PC via USB cable and check if the drive of “ NODE_F411RE (E:)” is created.

- To load the program onto the MCU, copy the downloaded binary file to the drive “ NODE_F411RE (E:)”. If the program is loaded successfully then LED(LD1) will be green light.

- LED (LD2) should be turned ON when the button (USER B1) is pressed.

B. GPIO Register tutorial

- Open the program ‘Keil uVision5’ and create a new project. Name the project as ‘Tutorial_LED’. Refer to ‘Tutorial0_Project Start’ if you need help.

- Create a new item called ‘main.c’

- Copy and paste the given source code on ‘main.c’. This is an example code of turning ON LED without button input. You will modify this code to include the push-button input. Do not worry if you do not understand what this code means yet. You will learn one by one in the following few weeks.

- Compile(F7) and flash(F8) the source code on to the MCU board.

- Verify the program by checking if the LED is turned ON.

IV. Exercise/Demo

- Modify the code to turn ON LED for different speed and check any difference

  - Low High / Very High Speed

- Modify the code to turn Toggle LED with a push-button input.

  - Button Input: Port C Pin 15 / Pull-up

  - Draw a flowchart or pseducode for this program

  - Check the output pin with oscilloscope and observe how the signals change with input button

- Create user defined functions for GPIO control (due by end of next lab)

- Create your own function to initialize the clock setting, and control GPIO. Below are examples of the functions. Attach your own codes in Appendix.

| Source File | Function |
| --- | --- |
| myRCC.h | void RCC_HSI_init () |
| myGPIO.h | void GPIO_init(GPIO_TypeDef *Port, int Pin, int I/OMode) |
|  | void GPIO_otype(GPIO_TypeDef *Port, int Pin, int Otype) |
|  | void GPIO_pupd(GPIO_TypeDef *Port, int Pin, int Pupd) |
|  | void GPIO_write(GPIO_TypeDef *Port, int Pin, uint8 Out) |
|  | uint8 GPIO_read(GPIO_TypeDef *Port, int Pin) |

V. Questions

- Find out a typical solution for software debouncing and hardware debouncing. What method did this NUCLEO board used for the push-button(B1)?

- What are the differences between open-drain and Push-pull for output pin?

- Which method of pullup/pulldown did this NUCLEO board used for LED and Button?

- What is the GPIO output speed? Explain the advantage of low speed for LED output than high speed, if there is any.

Appendix

- Pin Configuration of NUCLE-F401RE

- LED/Button Circuit Diagram